REMOVABLE MUZZLEBRAKE

All Rifles, Inc. muzzlebrakes are removable.  Most people choose to never remove the brake.  If you do decide to remove the brake you should order a thread protector cap.  To remove the brake:

- Place the rubber gripper around the brake.

- Place the brake with the gripper in a vise and carefully tighten the vise.  It is possible to crush the brake.  Only get the vise snug.

- Turn the rifle off the brake.  This is a standard right-handed thread and the rifle should be turned counter-clockwise to remove.

- Place the thread protector cap on the thread end of the barrel.  This is very important – if these threads become crushed, the brake will not go back on correctly.

TO PUT THE BRAKE ON

- Remove the thread protector cap – place the rubber gripper around the cap and use either a vise or channel grips to remove the cap.  This again is a standard right-handed thread and should be removed counter-clockwise.

- Hand place the brake on the end of the barrel and hand-tighten.

- Place rubber gripper on the brake and place covered brake in vise.  Tighten vise snuggly – but not over tightly.

- Tighten the rifle on the brake.  It is best to use a sharp turning motion instead of continually tightening down.