RIFLE CLEANING PROCEDURE

Generally when a rifle becomes inaccurate it is due to fouling in the bore.  This happens at a different rate for each rifle.  A good rule of thumb is to shoot the rifle until the accuracy is affected then divide that number by half, the resulting number is that particular rifles cleaning schedule.  Example – if accuracy drops after 30 shots, then you should clean that rifle after 15 shots to obtain the greatest accuracy.  One of the factors for the rate in which a rifle fouls is the type of bullet.  Some bullets just foul more than others as well as some cartridges, especially ones with higher velocities will foul faster.  Some barrels foul to the point of unsatisfactory accuracy in as few as 10 shots and others the number is more like 50.  Each barrel, even from the same manufacturer can vary this much.

CLEANING

- Remove bolt and insert bore guide to prevent damage to throat and to keep cleaning solvents out of the trigger assembly.

- Use a nylon bristled brush with proper bore diameter.

- Wrap a cotton patch around the brush and saturate patch with Remington Bore Cleaner.  If the brush/patch is too large to push through the bore, cut the patch in half diagonally.

- Push brush and patch through bore with a back and forth motion to muzzle 10 times, being careful not to damage the crown of the muzzle with the brush and rod.

- Repeat step 4 with new patch saturated in Remington Bore Cleaner.  May need to be repeated 1-3 more times.

- Place clean patch on nylon brush and saturate with Hoppes Bench Rest Solvent – run through bore about 4 times.  Repeat with new patch and solvent until all black color is gone.

- Put new patch on nylon brush and run through bore.  Repeat until patch is dry.  Usually this will take 2-3 patches.

- For prolonged storage you might choose to leave a light coat of Hoppes Bench Rest Solvent in bore but the rifle will need to be dry patched before shooting.

- If the metal finish is Black Teflon, there is no need to oil the outside of the barrel or receiver parts.  If the metal finish is Matte Stainless, we recommend a light spraying of Birchwood-Casey’s Sheath Rust Preventative and wiping down.

The chemicals in these products are very aggressive.  Take precautions not to get the liquid on the stock or exterior metal.  It will damage and/or discolor the stock finish and metal finish.  Use in a well-ventilated area to avoid breathing fumes.